MEMO

| TO: | Olivia Dias, Current Planning Manager Community Development Department |
| --- | --- |
| FROM: | Glenn J. Davis, PE, CFM, Chief Development Engineer Public Works Department |
| DATE: | June 4, 2021 |
| SUBJECT: | PUBLIC WORKS RECOMMENDATIONS SUB-ADJ21-06 (21-103682-LD) 4400 BLOCK OF BATTLE CREEK ROAD SE 177-LOT SUBDIVISION |

PROPOSAL

A consolidated application for a 177-lot phased subdivision (Coburn Grand View Estates) northeast of the intersection of Kuebler Boulevard SE and Battle Creek Road SE, containing the following requests:

- A subdivision tentative plan to divide approximately 32.55 acres into 177 lots ranging in size from approximately 4,400 square feet to approximately 13,500 square feet, including Alternative Street Standards request to increase the street grade from 12 percent to 15 percent for nine street intervals less than 200 feet long, and 52-foot right-of-way, where 60 feet is required for three proposed streets;

- Zoning Adjustment application to increase the Maximum Lot Depth standard of 300 percent width for lots 34-53, lots 55-62, and lots 75-77 to approximately 700 percent; adjust Lot Depth for double frontage lots 75, 89-93, and 101-103 from 120 feet to 118 feet; reduce the minimum lot width in the RM-II zone from 20 feet to 16.50 feet for Lots 35, 38, 41, 44, 47, 48, 51, 52, 56, 59, 62, and 77; adjust the Lot Depth of lot 27 from 70 feet to 30 feet; and adjust the minimum lot width for lots 35, 38, 41, 44, 47, 48, 51, 52, 56, 59, and 62 from 20 feet to 16.5 feet.

The subject properties are approximately 32.5 acres, zoned RA (Residential Agriculture) and RM-II (Multiple Family Residential) and located on the 4400 Block of Battle Creek Road SE (Marion County Assessor Map and Tax Lot Numbers 083W12B / 1600, 083W12C / 700 and 083W11D / 200, 400, 601).

RECOMMENDED CONDITIONS OF PLAT APPROVAL

As a condition of final plat approval for all phases:

- Provide an engineered stormwater design pursuant to SRC 71 and PWDS to accommodate future impervious surface on all proposed lots, including stormwater facilities needed to serve new streets.

- All lots within the S-1 service level shall be served by S-1 water mains, and all lots within the S-2 service level shall be served by S-2 water mains.

- Dedicate a 10-foot public utility easement along the street frontage of all internal streets.

- Construct internal streets to Local street standards, including property line sidewalks and installation of street trees.  Proposed Streets K, M, and N shall be a minimum of 52 feet in width, which includes a four-foot planter strip to accommodate street trees and sidewalks adjacent to one foot from property line. All other streets shall meet Local street standards.

- The street areas shown on the Slope Exception Areas plan are approved as an alternative street standard pursuant to SRC 803.065 with slopes not exceeding 15 percent.

- Construct Coburn Reservoir, Boone Road S-1 Pump Station, and S-1 connecting water mains as indicated in Water System Master Plan; or pay a Temporary Access Fee (TAF) pursuant to SRC 200.080(a). The TAF shall be $12,000 per acre of land being developed within the S-1 service area as proportional payments within each section as specified below.

- Reserve property for dedication of neighborhood park facility not less than 6.5 acres in size.

- Obtain applicable State and Federal permits as indicated by the Department of State Lands (DSL).

- Section A shall be the first phase.

As a condition of final plat for Section A:

- Construct a realigned intersection of Battle Creek Road SE and Reed Road SE to Minor Arterial standards pursuant to PWDS, and acquire and/or convey land for dedication of right-of-way as needed to construct the improvements.  The new intersection shall include a westbound to southbound left-turn lane on Reed Road SE, a southbound to eastbound left-turn lane on Battle Creek Road SE, a northbound to eastbound right-turn lane on Battle Creek Road SE, and a pedestrian island on Battle Creek Road SE along the south side of the intersection. If off-site right-of-way is needed to construct the improvements and the applicant is unable to acquire the right-of-way from adjacent property owner(s), then the applicant shall follow the procedures established in SRC 200.050.

- Along Reed Road SE from Battle Creek Road SE to the north line of the subject property, construct a minimum 23-foot-wide half street improvement to Minor Arterial standards within a minimum 36-foot-wide half-width right-of-way.  The improvements shall include on-street parking and tapers.

- Along Battle Creek Road SE from Reed Road SE to M Street SE, construct a 23-foot-wide half-street improvement to Minor Arterial standards within a minimum 36-foot-wide half-width right-of-way.

- Construct M Street SE from Battle Creek Road SE to N Street SE.

- Construct multi-modal trails to the proposed park within Section A as shown on tentative subdivision plan to PWDS.  Where topography requires mid-block walkways to be constructed with stairs, the facility shall be owned and maintained by the homeowners’ association.

- Enter into a deferral agreement to construct a pedestrian access route along missing sections of sidewalk on the west side of Battle Creek Road SE from Reed Road SE to Forsythe Drive SE, to be constructed in conjunction with the Apartment Site project.

- Construct a minimum 18-inch S-1 water main in Reed Road SE from Strong Road SE to J Avenue SE.

- Construct a minimum 18-inch S-1 water main in Section A from the intersection of Reed Road SE and J Avenue SE to the westerly terminus of G Street SE, and to the southerly Phase boundary in K Street SE.

- Construct a minimum 12-inch S-1 water main in Reed Road SE from J Avenue SE to Battle Creek Road SE, and in Battle Creek Road SE to M Street SE.

- Pay $191,849 as the proportional amount of the water TAF.

As a condition of final plat for Section B:

- Construct M Street from N Street to the east line of Section B.

- Construct a minimum 18-inch water main in K Street SE within Section B from its terminus in Section A to the southern property line of lot 106.

- Pay $14,038 as the proportional amount of the water TAF.

- Pay a temporary access fee of $13,680 as proportional mitigation toward the future signal and interconnect located at Reed Road SE and Battle Creek Road SE.

- Construct multi-modal trails to the proposed park within Section B as shown on tentative subdivision plan to PWDS.

- Convey lot 106 to the City for future construction of Coburn Reservoir.

As a condition of final plat for Section C

- Convey land for dedication of right-of-way to equal a half-width of 30 feet from centerline along the entire frontage of Strong Road SE.

- Construct a 15-foot-wide half-street improvement on the development side of centerline and a 15-foot-wide pavement widening on the opposite side of centerline along Strong Road SE.

- Construct a minimum 18-inch S-1 water main within Section C from its terminus in Section A to the intersection of Strong Road SE and A Street SE.

- Pay $86,566 as the proportional amount of the water TAF.

As a condition of final plat for Section D:

- If off-site easements are needed because of topographic constraints for stormwater discharge from the northerly terminus of I Avenue SE and the applicant is unable to acquire easements from adjacent property owner(s), then the applicant shall follow the procedures established in SRC 200.050.

- Pay $79,547 as the proportional amount of the water TAF.

FACTS AND FINDINGS

Water

- Existing Conditions

  - The subject property is located within the S-1 and S-2 water service levels.

  - The nearest adequate S-1 water main is located in Reed Road SE at Strong Road SE, approximately 2,000 feet north of the subject property.

  - The nearest S-2 water main is located in Battle Creek Road SE between Pringle Road SE and Kuebler Boulevard SE.

Sanitary Sewer

- Existing Conditions

- A 15-inch sewer line is located in Reed Road SE.

- A 24-Inch sewer line is located in 27th Avenue SE.

Storm Drainage

- Existing Condition—Drainage patterns in this area are generally conveyed into Pringle Creek. The property is on a ridge in which the drainage flows to the northwest toward the West Middle Fork of Pringle Creek, northeast toward Strong Road SE, or southeast toward Kuebler Boulevard SE.

Streets

- Reed Road SE

  - Standard—This street is designated as a Minor Arterial street in the Salem TSP. The standard for this street classification is a 46-foot-wide improvement within a 72-foot-wide right-of-way.

  - Existing Condition—This street has an approximate 20-foot improvement within a 
40-foot-wide right-of-way abutting the subject property.

- Battle Creek Road SE

  - Standard—This street is designated as a Minor Arterial street in the Salem TSP. The standard for this street classification is a 46-foot-wide improvement within a 72-foot-wide right-of-way.

  - Existing Condition—This street has an approximate 20- to 46-foot improvement within a 60- to 72-foot-wide right-of-way abutting the subject property.

- Kuebler Boulevard SE

  - Standard—This street is designated as a Parkway street in the Salem TSP. The standard for this street classification is an 80-foot-wide improvement within a 120-foot-wide right of way.

  - Existing Condition—This street has an approximate 70- to 80-foot improvement within a 120- to 150-foot-wide right-of-way abutting the subject property. The parkway was previously approved for an alternative street standard to allow the 70-foot improvement width.

  - Access control—No driveway access will be granted onto Kuebler Boulevard SE.

- Strong Road SE

  - Standard—This street is designated as a Collector street in the Salem TSP. The standard for this street classification is a 34-foot-wide improvement within a

60-foot-wide right-of-way.

  - Existing Condition—This street has an approximate 20- to 30-foot improvement within a 40- to 60-foot-wide right-of-way abutting the subject property.

Parks

No parks facilities are available to serve the proposed development. The Comprehensive Parks System Master Plan shows that a Neighborhood Park (NP 32) and Urban Park (UP 3) are planned on or near the subject property.

CRITERIA AND FINDINGS

The following code references indicate the criteria that must be found to exist before an affirmative decision may be made. The applicable criteria and the corresponding findings are as follows:

SRC 205.010(d)(1)—The tentative subdivision plan complies with the standards of this Chapter and with all applicable provisions of the Unified Development Code, including, but not limited to the following:

1.	Lot standards, including, but not limited to, standards for lot area, lot width and depth, lot frontage, and designation of front and rear lot lines;

2.	City infrastructure standards; and

3.	Any special development standards, including, but not limited to floodplain development, special setbacks, geological or geotechnical analysis, and vision clearance.

Findings—The applicant shall provide the required field survey and subdivision plat per Statute and Code requirements outlined in the Oregon Revised Statutes (ORS) and SRC. If said documents do not comply with the requirements outlined in ORS and SRC, and as per SRC Chapter 205, the approval of the subdivision plat by the City Surveyor may be delayed or denied based on the non-compliant violation. It is recommended the applicant request a pre-plat review meeting between the City Surveyor and the applicant’s project surveyor to ensure compliance with ORS 672.005(2)(g)&(h), 672.007(2)(b), 672.045(2), 672.060(4), Oregon Administrative Rules 850-020-0015(4)&(10), 820-020-0020(2), and 820-020-0045(5).

Public Works staff has reviewed the Flood Insurance Study and Flood Insurance Rate Maps and has determined that no floodplain or floodway areas exist on the subject property.

The Salem-Keizer Local Wetland Inventory shows that there are wetland channels and/or hydric soils mapped on the property. The applicant should contact the Oregon Department of State Lands to verify if any permits are required for development or construction in the vicinity of the mapped wetland area(s). Wetland notice was sent to the Oregon Department of State Lands pursuant to SRC 809.025.  The applicant shall obtain applicable State and Federal permits as indicated by the Department of State Lands (DSL).

According to the City’s adopted landslide hazard susceptibility maps and SRC Chapter 810 (Landslide Hazards), there are areas of landslide susceptibility on the subject property. There are 3 mapped site points for the property pursuant to SRC Chapter 810. The proposed subdivision adds 3 activity points to the proposal, which results in a total of 6 points. Therefore, the proposed subdivision is classified as a moderate landslide risk and requires a geologic assessment. A geologic assessment, prepared by Redmond Geotechnical Services and dated July 20, 2018, was submitted to the City of Salem. This assessment demonstrates the subject property could be subdivided and developed with single-family dwellings, without increasing the potential for slope hazard on the site or adjacent properties.

SRC 205.010(d)(3)—Development within the tentative subdivision plan can be adequately served by City infrastructure.

Findings—The nearest adequate water main is located in Reed Road SE at Strong Road SE, approximately 2,000 feet north of the subject property. The applicant shall be required to construct a minimum 18-inch S-1 water main in Reed Road SE from Strong Road SE to J Avenue SE, and through the subject property to Strong Road SE at A Avenue SE, and to the Coburn Reservoir site.  A 12-inch main is required in Reed Road SE from J Avenue SE to Battle Creek Road SE, and in Battle Creek Road SE from Reed Road SE to M Street SE.

The Water System Master Plan specifies that the subject property is to be served by the future Coburn Reservoir and future Boone Road S-1 pump station. The subject property surrounds the site of the future Coburn Reservoir (tax lots 083W12C00701 and

083W11D00101). The existing reservoir site does not provide two acres of land needed for construction of the reservoir. Therefore, the applicant shall be required to convey lot 106 for future construction of Coburn Reservoir.

Temporary capacity is available in the Mill Creek Reservoir and Deer Park Pump Station through payment of a temporary access fee pursuant to SRC 200.080(a). In lieu of constructing Coburn Reservoir and Boone Road Pump Station, the applicant has the option of entering into a temporary facilities access agreement with the City and paying a temporary access fee (TAF). The TAF for this area of the S-1 service area is based on approximately 9 million dollars of improvements needed to serve 750 acres of undeveloped land, totaling $12,000 per acre.  The total S-1 area for the subject property is 31.0 acres; therefore, the TAF shall be a total of $372,000. The TAF has been apportioned to each section based on the number of lots being created in the S-1 area as shown in the following table:

| Section | S-1 Lots | Percentage | TAF Amount |
| --- | --- | --- | --- |
| A | 82 | 51.5 | $191,849 |
| B | 6 | 3.8 | $14,038 |
| C | 37 | 23.3 | $86,566 |
| D | 34 | 21.4 | $79,547 |

All lots will be served by sewer as depicted on the applicant’s preliminary utility plan.   As a condition of sewer service, all developments will be required to provide public sewers to adjacent upstream parcels. This shall include the extension of sewer mains in easements or rights-of-way across the property to adjoining properties, and across the street frontage of the property to adjoining properties when the main is located in the street right-of-way. If off-site easements are needed and the applicant is unable to acquire easements from adjacent property owner(s), then the applicant shall follow the procedures established in SRC 200.050.

The proposed development is subject to the revised SRC Chapter 71 and PWDS as adopted in Administrative Rule 109, Division 004. To demonstrate the proposed parcels can meet the PWDS, the applicant shall provide an engineered tentative stormwater design to accommodate future impervious surface on all proposed lots. The applicant’s engineer has submitted a preliminary design report that demonstrates compliance with PWDS. Prior to final plat of all subdivision phases, the applicant shall provide an engineered stormwater design pursuant to SRC Chapter 71 and PWDS to accommodate future impervious surface on all proposed lots, including stormwater facilities needed to serve new streets.  If off-site easements are needed because of topographic constraints for stormwater discharge from the northerly terminus of I Avenue SE and the applicant is unable to acquire easements from adjacent property owner(s), then the applicant shall follow the procedures established in SRC 200.050

All public and private City infrastructure proposed to be located in the public right-of-way shall be constructed or secured per SRC 205.035(c)(7)(B) prior to final plat approval. Any easements needed to serve the proposed parcels with City infrastructure shall be shown on the final plat.

SRC 205.010(d)(4) and SRC 205.0010(d)(5)—The street system in and adjacent to the tentative subdivision plan conforms to the Salem Transportation System Plan. The street system in and adjacent to the tentative subdivision plan is designed so as to provide for the safe, orderly, and efficient circulation of traffic into, through, and out of the subdivision.

Findings—Battle Creek Road SE, Reed Road SE, and Strong Road SE abut the subject property and do not meet the current standard for their classification of street. Pursuant to SRC 200.055(d), boundary street improvements and right-of-way dedication are required along Battle Creek Road SE, Reed Road SE, and Strong Road SE based on their street classification. Reed Road SE and Battle Creek Road SE are designated as Minor Arterial streets in the TSP. Reed Road SE abuts the northwestern boundary of the subject property and Battle Creek Road SE the southern property line. Currently, Battle Creek Road SE and Reed Road SE do not meet Minor Arterial standards. The applicant will be required to convey for dedication half-width right-of-way equal to 36 feet from center line and construct a half-street improvement along the entire frontage to a Minor Arterial standard. Strong Road SE is designated as a Collector street in the Salem TSP and does not meet current standards for right-of-way and street improvement widths.  The applicant shall convey land for dedication of right-of-way to equal a half-width of 30 feet from centerline along the entire frontage of Strong Road SE and construct a 15-foot-wide half-street improvement on the development side of centerline and a 15-foot-wide pavement widening on the opposite side of centerline along Strong Road SE (SRC 803.040).

A realignment of the intersection at Battle Creek Road SE and Reed Road SE is warranted because the intersection angle does not meet PWDS.  The applicant shall be required to construct a realigned intersection of Battle Creek Road SE and Reed Road SE to Minor Arterial standards pursuant to PWDS and acquire and/or convey land for dedication of right-of-way as needed to construct the improvements (SRC 803.025).  The new intersection shall include a westbound to southbound left-turn lane on Reed Road SE, a southbound to eastbound left-turn lane on Battle Creek Road SE, a northbound to eastbound right-turn lane on Battle Creek Road SE, and a pedestrian island on Battle Creek Road SE along the south side of the intersection. If off-site right-of-way is needed to construct the improvements and the applicant is unable to acquire the right-of-way from adjacent property owner(s), then the applicant shall follow the procedures established in SRC 200.050.

The application materials request several alternate street standards pursuant to SRC 803.065 for internal streets.  These street sections meet the criteria for alternative street standards as described below:

- A few short sections of internal streets (I Avenue, J Avenue, and K Street) require a street grade up to 15 percent as shown on the Slope Exceptions Plan.  These street sections are warranted because of topographic constraints.

- K Street from J Avenue to the east line of the subject property, M Street from J Avenue to O Street, and N Street from the north line of the subject property to M Street, are proposed for a 52-foot-wide right-of-way.  The minimum right-of-way for these streets shall be 52 feet with a minimum 4-foot-wide landscape strip between the curb and sidewalk.  These alternative standards are warranted because of topographic constraints.

- K Street, M Street, and N Street have block lengths exceeding 600 feet.  The applicant shall be required to install mid-block pedestrian accessways between M Street and N Street and between M Street and K Street.  These longer block lengths are warranted because of topographic constraints.

Pursuant to UG Preliminary Declaration 17-03, the applicant proposed a neighborhood park within the development along Reed Road SE.  The standard Minor Arterial street section includes bike lanes, but not parking.   Neighborhood parks are generally located on local streets.  Neighborhood park standards do not generally include a separate parking lot, but instead are served by on-street parking.  Therefore, the applicant is required to provide on-street parking along Reed Road SE in order to provide safe and convenient bicycle and pedestrian access from within the subdivision to adjacent residential areas to neighborhood activity centers within one-half mile of the development.

SRC 205.010(d)(6)—The tentative subdivision plan provides safe and convenient bicycle and pedestrian access from within the subdivision to adjacent residential areas and transit stops, and to neighborhood activity centers within one-half mile of the development. For purposes of this criterion, neighborhood activity centers include, but are not limited to, existing or planned schools, parks, shopping areas, transit stops, or employment centers.

Findings—No sidewalk exists along portions of the west side of Battle Creek Road SE from Reed Road SE to Forsythe Drive SE. Sidewalk improvements are required to provide safe pedestrian access to the proposed park being developed in conjunction with the proposed development.  The applicant shall construct a pedestrian access route along missing sections of sidewalk on the west side of Battle Creek Road SE from Reed Road SE to Forsythe Drive SE in conjunction with the Apartment Site project.

No parks facilities are available to serve the proposed development. The Comprehensive Parks System Master Plan shows that a Neighborhood Park (NP 32) and Urban Park (UP 3) are planned on or near the subject property. The applicant shall reserve property for dedication of neighborhood park facility NP-32 based on sizing criteria established in SRC 200.075(b).  The City is in contract to purchase a 7.35-acre portion of the subject property for a future park, which would satisfy this condition.

SRC 205.010(d)(7)—The tentative subdivision plan mitigates impacts to the transportation system consistent with the approved Traffic Impact Analysis (TIA), where applicable.

Findings—The applicant submitted a TIA from Associated Transportation Engineering & Planning, Inc. on June 20, 2018.  The TIA recommends additional turn lanes to be constructed at the intersection of Battle Creek Road SE and Reed Road SE.  The Assistant City Traffic Engineer has determined that additional mitigation measures are necessary based on the volume of traffic being generated by the development.

Based on the TIA data, the applicant shall be required to construct a realigned intersection of Battle Creek Road SE and Reed Road SE to Minor Arterial standards pursuant to PWDS, and acquire and/or convey land for dedication of right-of-way as needed to construct the improvements.  The new intersection shall include a westbound to southbound left-turn lane on Reed Road SE, a southbound to eastbound left-turn lane on Battle Creek Road SE, a northbound to eastbound right-turn lane on Battle Creek Road SE, and a pedestrian island on Battle Creek Road SE along the south side of the intersection. The reconstruction and alignment of the intersection is needed to provided safe turning movement onto Reed Road SE for the additional 177 dwellings.

A future signal will be warranted in the future at the intersection of Battle Creek Road SE and Reed Road SE.  Based on the TIA, the proposed development will add 32 PM peak hour vehicles to the intersection at Battle Creek Road SE and Reed Road SE, which is a 3.04 percent increase because the intersection currently has 1,056 vehicles at build-out during the PM peak hour.  The applicant is required to pay a temporary access fee of 3.04 percent of the future signal and interconnect cost of $450,000, or $13,680.

Prepared by: Jennifer Scott, Program Manager

cc: File